Guide to Azure Virtual Networks (vNET)

Azure Virtual Network (VNet) is the fundamental building block for your private network in Azure. It is a logical isolation of the Azure cloud dedicated to your subscription. With a VNet, you can provision and manage private virtual networks and connect them to other VNets or to your on-premises IT infrastructure.

A VNet allows you to create a secure, isolated environment for your resources. You can control the flow of traffic, define IP address ranges, and create network segments to organize your cloud infrastructure.

1. Azure Virtual Network (VNet)

A VNet is a virtualized representation of a network in the cloud. It's a logically isolated network in Azure where you can launch Azure resources.

- Key Characteristics:

  - IP Address Space: A VNet requires you to define a private IP address range using CIDR notation (e.g., 10.0.0.0/16). All resources within this VNet will be assigned an IP address from this range.

  - Regions: A VNet is scoped to a single Azure region. While you can connect VNets across regions using peering, a single VNet cannot span multiple regions.

  - Isolation: A VNet provides complete isolation. By default, resources in one VNet cannot communicate with resources in another VNet.

  - Internet Connectivity: All resources within a VNet can, by default, communicate outbound to the internet. Inbound traffic from the internet is blocked by default unless you explicitly allow it.

Lab Session: Creating an Azure VNet

This lab will guide you through creating a basic VNet in Azure.

Detailed Steps:

- Log in to the Azure Portal: Open a web browser and go to https://portal.azure.com. Log in with your Azure credentials.

- Search for Virtual Networks: In the search bar at the top of the portal, type "Virtual networks" and select it from the services list.

- Create a New VNet: Click the + Create button.

- Basics Tab:

  - Subscription: Select your Azure subscription.

  - Resource group: Click "Create new" and enter a name like vnet-rg. A resource group is a container that holds related resources for an Azure solution.

  - Name: Enter a unique name for your VNet, such as my-first-vnet.

  - Region: Select a region close to you (e.g., East US).

- IP Addresses Tab:

  - Here you define the IP address space for your VNet. Azure automatically suggests a default. You can keep it as 10.0.0.0/16 or change it.

  - A /16 CIDR block provides over 65,000 IP addresses. This is a good starting point for a simple VNet.

  - You will see a default subnet already listed. This is fine for now. We will work with subnets in the next section.

- Review + create: Review the settings. If everything looks correct, click the Create button. The deployment will begin and should complete in a few minutes.

You have now successfully created a foundational private network in Azure.

2. Subnets

Detailed Notes

A subnet is a range of IP addresses within a VNet. It is a logical division of your VNet's address space. Subnets are used to segment the network within a VNet for organization and security. All resources (like virtual machines) within a VNet must be deployed into a subnet.

- Why use subnets?

  - Network Segmentation: Subnets allow you to logically group resources. For example, you could have a WebTier subnet for your front-end web servers and a AppTier subnet for your application servers.

  - Security Boundaries: Subnets are the primary boundary for applying security controls like Network Security Groups (NSGs).

- How do subnets work?

  - The total IP address range of a VNet is broken down into smaller, non-overlapping ranges for each subnet.

  - Azure reserves the first four and last IP addresses in each subnet for internal use. For example, a /24 subnet has 256 IP addresses, but only 251 are available for your resources.

Lab Session: Creating Subnets within a VNet

This lab will show you how to create new subnets within the VNet you created earlier.

Detailed Steps:

- Navigate to your VNet: In the Azure Portal search bar, type my-first-vnet and select it.

- Go to Subnets: In the VNet's left-hand menu, under Settings, select Subnets. You will see the default subnet that was created with your VNet.

- Add a New Subnet: Click the + Subnet button at the top.

- Subnet Details:

  - Name: Give it a meaningful name, such as my-web-subnet.

  - Address range: Enter an address range that is a subset of your VNet's address space. For example, if your VNet is 10.0.0.0/16, you could use 10.0.1.0/24. The CIDR prefix must be between /16 and /29.

- Save: Click the Save button at the bottom.

You have now segmented your virtual network into a new, smaller subnet. You can repeat this process to create additional subnets for your application or database tiers.

3. Network Security Groups (NSG)

Detailed Notes

A Network Security Group (NSG) is a virtual firewall that filters inbound and outbound network traffic to and from Azure resources. An NSG contains a list of security rules that allow or deny traffic based on:

- Priority: Rules are processed in order of priority (from lowest number to highest). Once a rule matches, the processing stops.

- Source / Destination: IP addresses, CIDR blocks, or service tags.

- Direction: Inbound or outbound.

- Protocol: TCP, UDP, or Any.

- Port Range: The port or port range for the traffic.

- Action: Allow or Deny.

You can associate an NSG with a subnet or with a single network interface (NIC). When associated with a subnet, all resources in that subnet are protected by the NSG's rules.

Lab Session: Creating and Associating an NSG

This lab will guide you through creating an NSG with a rule to allow RDP access and then associating it with a subnet.

Detailed Steps:

- Create an NSG: In the Azure Portal, search for "Network security groups" and click + Create.

  - Subscription: Select your subscription.

  - Resource group: Use the existing vnet-rg.

  - Name: Name it my-web-subnet-nsg.

  - Region: Select the same region as your VNet.

  - Click Review + create and then Create.

- Add a Security Rule: Once the NSG is created, navigate to it in the portal. In the left-hand menu, under Settings, select Inbound security rules.

  - Click + Add.

  - Source: Any

  - Source port ranges: *

  - Destination: Any

  - Destination port ranges: 3389 (This is the standard port for RDP).

  - Protocol: TCP

  - Action: Allow

  - Priority: 100 (A low number gives it a high priority).

  - Name: Allow-RDP-Inbound

  - Click Add.

- Associate NSG with a Subnet:

  - Go back to your VNet (my-first-vnet).

  - In the left-hand menu, select Subnets.

  - Click on my-web-subnet.

  - In the dropdown for Network security group, select the NSG you just created (my-web-subnet-nsg).

  - Click Save.

Now, any virtual machine you place in the my-web-subnet will be protected by the NSG, and RDP traffic on port 3389 will be explicitly allowed.

4. Application Security Groups (ASG)

Detailed Notes

An Application Security Group (ASG) allows you to group resources (like virtual machines) and define network security rules based on these groups. This is a powerful feature that simplifies NSG management. Instead of defining rules with specific IP addresses, you can use the ASG name.

- Why use ASGs?

  - Simplified Rule Management: You no longer need to update NSG rules when you add or remove virtual machines. You just add the new VM's NIC to the ASG.

  - Improved Readability: Rules become much easier to understand. For example, "Allow traffic from WebServers to DatabaseServers" is much clearer than a rule with a list of IP addresses.

Lab Session: Using ASGs in NSG Rules

This lab demonstrates how to use an ASG to create a more manageable security rule.

Detailed Steps:

- Create a New ASG:

  - In the Azure Portal, search for "Application security groups" and click + Create.

  - Subscription: Select your subscription.

  - Resource group: Use vnet-rg.

  - Name: Name it my-web-servers-asg.

  - Region: Same as your VNet.

  - Click Review + create and then Create.

- Create a New NSG Rule Using an ASG:

  - Go to the my-web-subnet-nsg you created earlier.

  - In the left-hand menu, select Inbound security rules.

  - Click + Add.

  - Source: Change this from Any to Application security group.

  - Source application security group: Select my-web-servers-asg from the dropdown.

  - Source port ranges: *

  - Destination: Any

  - Destination port ranges: 80, 443 (HTTP and HTTPS).

  - Protocol: TCP

  - Action: Allow

  - Priority: 200

  - Name: Allow-Web-Inbound

  - Click Add.

- Associate a VM's NIC with the ASG (Conceptual):

  - Now, if you had a virtual machine, you would go to its network interface (NIC) settings.

  - Under the NIC's properties, you would find Application security groups and add your my-web-servers-asg to it.

  - Any new VM you want to be part of this group just needs to be added to the ASG. The NSG rule will automatically apply to it without any IP address changes.

5. VNet Peering

Detailed Notes

VNet Peering connects two Azure VNets. Once peered, the VNets appear as a single, contiguous network. Traffic between resources in the peered VNets is routed privately through the Microsoft backbone network. This means the communication is not exposed to the public internet and there's no need for a VPN gateway.

- Key Concepts:

  - Intra-Region Peering: Connecting VNets within the same Azure region. This provides high-bandwidth, low-latency connectivity.

  - Global VNet Peering: Connecting VNets across different Azure regions.

  - Non-Transitive: VNet peering is non-transitive. If VNet A is peered with VNet B, and VNet B is peered with VNet C, VNet A and VNet C are not automatically connected.

Lab Session: Setting up VNet Peering

This lab will show you how to peer two VNets. We will assume you have a second VNet named my-second-vnet with an IP address space of 10.1.0.0/16.

Detailed Steps:

- Create a Second VNet: Create a second VNet named my-second-vnet with an IP address space of 10.1.0.0/16 in the same or a different region. Make sure it's in a different IP range from my-first-vnet.

- Start the Peering from VNet 1:

  - Go to my-first-vnet in the Azure Portal.

  - In the left-hand menu, under Settings, select Peerings.

  - Click + Add.

- Configure the Peering:

  - Name of the peering from my-first-vnet to my-second-vnet: my-first-vnet-to-my-second-vnet

  - Virtual network deployment model: Leave as Resource Manager.

  - Subscription: Select your subscription.

  - Virtual network: Select my-second-vnet.

  - Name of the peering from my-second-vnet to my-first-vnet: my-second-vnet-to-my-first-vnet

  - Allow VNet Access: Leave as enabled.

- Create the Peering: Click Add. Azure will automatically create the peering connections on both sides.

- Verify the Peering:

  - Once the deployment is complete, go back to the Peerings page for my-first-vnet.

  - You should see a new peering connection with a Peering status of Connected.

  - If you go to my-second-vnet and look at its Peerings, you will also see a connected status.

Traffic can now flow privately between the two VNets.

6. NAT Gateway

Detailed Notes

An Azure NAT Gateway is a managed service that provides outbound internet connectivity for virtual networks. It hides the private IP addresses of your virtual machines and presents a single, public IP address for all outbound connections.

- Why use a NAT Gateway?

  - Simplified Outbound Connectivity: It provides a single, scalable point of exit for a subnet. You don't need to assign a public IP address to every VM.

  - Security: It prevents inbound connections from the internet unless it's a response to an outbound connection. This enhances the security posture of your private resources.

  - Scalability: A NAT Gateway can scale automatically to handle a large number of outbound connections without needing any configuration.

Lab Session: Creating and Associating a NAT Gateway

This lab will show you how to set up a NAT Gateway and associate it with a subnet to provide outbound internet access.

Detailed Steps:

- Create a Public IP Address: A NAT Gateway needs a public IP address to work.

  - In the Azure Portal, search for "Public IP addresses" and click + Create.

  - Subscription: Your subscription.

  - Resource group: vnet-rg.

  - Name: my-nat-public-ip.

  - IP SKU: Standard (Required for NAT Gateway).

  - Tier: Regional.

  - Click Review + create and then Create.

- Create the NAT Gateway:

  - In the Azure Portal, search for "NAT gateways" and click + Create.

  - Subscription: Your subscription.

  - Resource group: vnet-rg.

  - Name: my-nat-gateway.

  - Region: Same as your VNet.

  - Idle timeout: 4 minutes (Default, can be adjusted).

  - Public IP addresses: Select the my-nat-public-ip you just created.

- Associate with a Subnet:

  - Click the Subnet tab.

  - Select your VNet (my-first-vnet).

  - Select my-web-subnet from the list. This will associate the NAT Gateway with that subnet.

  - Click Review + create and then Create.

Any virtual machine placed in the my-web-subnet will now use the NAT Gateway's public IP address for all its outbound internet traffic.

Interview Questions:

WHAT IS A VIRTUAL NETWORK (VNET) IN AZURE AND HOW DOES IT WORK?

What are the key differences between a VNet and a traditional on-premise network?

How do you design subnetting within a VNet and why is it important?

What are the supported IP address ranges for VNets?

How is routing handled between subnets in the same VNet?

WHAT IS A NETWORK SECURITY GROUP (NSG) IN AZURE?

WHAT IS THE DIFFERENCE BETWEEN NSG RULES AND FIREWALL RULES?

WHAT ARE THE DEFAULT NSG RULES, AND CAN YOU OVERRIDE THEM?

HOW DO NSGS EVALUATE TRAFFIC (PRIORITY, DIRECTION, AND RULES)?

HOW CAN YOU APPLY NSGS — AT THE NIC LEVEL VS SUBNET LEVEL? WHAT’S THE IMPACT?

WHAT IS AN APPLICATION SECURITY GROUP (ASG)?

HOW DO ASGS SIMPLIFY NETWORK SECURITY MANAGEMENT?

CAN ASGS SPAN MULTIPLE VNETS? WHY OR WHY NOT?

HOW ARE ASGS USED IN COMBINATION WITH NSGS?

WHAT IS VNET PEERING AND HOW IS IT DIFFERENT FROM VPN GATEWAY?

WHAT ARE THE KEY BENEFITS OF VNET PEERING?

WHAT IS GLOBAL VNET PEERING, AND HOW DOES IT WORK?

WHAT ARE THE LIMITATIONS OF VNET PEERING?

CAN PEERED VNETS HAVE OVERLAPPING IP RANGES? WHY OR WHY NOT?